GHG Emission Reduction Calculation and Incremental Cost Analysis

for the GEF Efficient and Green Freight Transport Project

- Economic Analysis. Following GEF guidelines, an incremental cost analysis was conducted to assess the incremental costs and global environmental benefits of the Project scenario vis-à-vis business-as-usual (BAU) scenario.

- BAU Scenario. The national government has prioritized the development of multimodal freight transport and improvement of urban freight in the 13th FYP and other key policy directives in the transport sector. The local governments have accelerated the investment in freight infrastructure and services, such as intermodal hubs, logistics parks, freight railways, inland waterway, freight distribution centers, etc. These directives and investments will increase the capacity for intermodal transport and urban freight distribution, and attract some freight from road to cleaner modes of transport. However, due to inefficiencies in operations and policy/standardization barriers, the capacity may not be fully utilized despite the new infrastructure being provided.

- Incremental Costs. The incremental costs of this GEF project are associated with: (i) developing policies and strategies at the national level to support the development of multimodal transport and improve the efficiency of urban freight; (ii) developing analytical tools and policy framework to support the five pilots at the local level; and (iii) capacity building to promote the national strategies and policies and disseminate the pilot experiences.

- Global Environmental Benefits. The global environmental benefits of the Project intervention include two types: (i) direct GHG emission reduction from the pilot projects, such as modal shift from road transport to waterway and/or railway, reduced urban distribution trips due to consolidation, reduced empty-loaded freight trips, etc.; and (ii) indirect GHG emission reduction because of policy incentives and capacity building activities that promote the replication of successful pilots. A more detailed incremental reasoning by component is provided in Table 1.

- Two pilot projects were selected for detailed analysis of direct GHG emission reduction: the Bohai Bay Highway-Waterway Multimodal Transport Project in Yantai, and the Integrated Urban-Rural Distribution Project in Guangdong.

- Bohai Bay Highway-Waterway Multimodal Transport Project in Yantai, Shandong Province. In this project, the emission reduction is through “shift” approach, i.e. attracting more traffic from road to sea transport. In the BAU scenario, it is assumed that the number of trucks (both in roll-on-roll-off (RoRo) and drop-and-pull (DnP) mode) is increasing at the same rate (5%) annually, which is derived from the historic seaway transport data in the past 10 years. In the GEF scenario, the number of trucks in RoRo mode remains the same as BAU, while the share of DnP is expected to level up at the targeted growth rate.

- In the BAU scenario, with the most recent number of trucks by seaway in year 2016 and the normal growth rate (5%), the total number of trucks by seaway in the studied years could be estimated. With the same methodology, annual BAU ratio of DnP/total trucks is calculated as 2.1%, thus number of DnP trucks can be calculated as well.

Total trucks by seaway (BAU) = Baseline number of trucks by seaway × 1.05n

Number of DnP (BAU) = Total trucks by seaway (BAU) × BAU ratio

Number of RoRo (BAU) = Total trucks by seaway (BAU) × (1- BAU ratio)

where n denotes the number of years after the baseline year

- In the GEF scenario, given the targeted ratio of DnP over total sea transport in each year – provided by the ferry operator – and the number of RoRo trucks, the number of DnP trucks can be estimated. Thus the total number of trucks by seaway can be estimated.

Number of DnP (GEF) = Number of RoRo (BAU) × (targeted ratio/1-targeted ratio)

Total trucks by seaway (GEF) = Number of RoRo (BAU) + Number of DnP (GEF)

- The impact of GEF intervention is the increase of trucks using sea transport, particularly, RoRo.

Number of increased trucks by seaway = Total trucks by seaway (GEF) – Total trucks by seaway (BAU)

- The increased trucks by seaway were supposed to travel from Yantai to Dalian by road transport under the BAU scenario. The direct emission reduction resulted from the GEF intervention is thus calculated using the road transport distance and emission factor for trucks, which is 0.51 kg CO2/km.

Direct emission reduction = Number of increased trucks by seaway × road transport distance × Emission factor for trucks

- As more trucks are transported by seaway, number of ship trips may increase leading to additional emissions. Based on the operator’s information, one freight ship can carry 300 trucks per trip. The number of ship trips per day can thus be calculated and all numbers are rounded up.

Number of ship trips per day (BAU or GEF) = Total trucks by seaway (BAU or GEF) / 365 / 300

Increased ship trips per day = Number of ship trips per day (BAU) - Number of ship trips per day (GEF)

- Given the emission factor for ship fuel use is 3150 kg CO2/ ton, and the fuel use per ship per trip is 18 tons oil – provided by the ship operator – the increased CO2 by ship per year can be calculated.

Increased CO2 by ship per year = Increased ship trips per day × Oil use per ship per trip × Emission factor for ships × 365

- Each year the CO2 reduction is the difference between CO2 emission saved from road and CO2 emission increased from ship.

Direct CO2 reduction per year = Direct emission reduction from road – Increased CO2 by ship per year

Total CO2 reduction = 1nDirect CO2 reduction per year,  where n indicates years of GEF project

- The lifecycle of this pilot project is considered to be from 2020 to 2030. Based on the calculation in Table 2, the lifecycle CO2 emission reduction from the Yantai pilot is estimated to be 1.10 million tons.

- Integrated Urban-Rural Distribution Project in Guangdong Province. In the BAU scenario, the empty loaded rate of return trucks remains a slow decrease each year. With GEF intervention, the empty loaded rate of return trucks is expected to decrease rapidly. The direct emission reduction resulted from utilizing the return urban freight vehicles to replace the rural freight vehicles for delivering the rural products to the urban area. It is expected that 2000 urban freight vehicles will register in the information platform under the GEF scenario.

- Based on the information provided by the logistics operator, among the 2000 registered urban freight vehicles, 1467 vehicles are in 4.2 meter long and 533 are in 9.6 meter long. Also, trips per vehicle per month is 15 trips per month for each vehicle. Thus the loaded return trips from Qingyuan to Guangzhou can be calculated.

Loaded return trips per month = 15 × (1 – empty loaded rate) × number of pilot urban freight vehicle

- The capacity of urban freight vehicles are larger than the rural freight vehicles. It is assumed that (1) for the 4.2m-long pilot vehicle, one loaded return trip can save 4 trips for rural freight vehicles from Qingyuan to Guangzhou, and (2) for the 9.6m-long pilot vehicle, one loaded return trip can save 8 trips for rural freight vehicles from Qingyuan to Guangzhou. Taking the trip from Guangzhou back to Qingyuan into consideration, the total saved rural freight trips per month are doubled.

Saved rural freight trips per month = 2 × (4 × Loaded return trips per month_4.2m + 8 × Loaded return trips per month_9.6m)

- The average trip distance from Qingyuan to Guangzhou is 97km and the emission factor1 for small rural freight vehicle is 0.32kg CO2/km, the direct emission reduction from saved rural freight trips can be estimated.

Direct CO2 reduction per month = Saved rural freight trips per month × emission factor × average distance

- The lifecycle of this pilot project is considered to be from 2020 to 2030. Based on the calculation in Table 3, the lifecycle CO2 emission reduction from the GuangQing pilot is estimated to be 0.23 million tons.

- Incremental Cost Analysis. The direct CO2 emission reduction from these two pilots are 1.1 million tons (Yantai multimodal transport) and 0.23 million tons (Guangdong urban-rural distribution). Currently, MOT has approved 16 ongoing multimodal transport pilots; and China has more than 30 provinces that could adopt similar urban-rural distribution modality. With the issuance of national policies and guidelines to address existing impediments in multimodal transport and urban distribution, together with the capacity building and dissemination activities, it is expected than these two types of projects will have a replication effect of 10 and 20 times, respectively. Total global environmental benefits, both direct and indirect, is estimated to be 15.6 million tons. The incremental cost of the Project is estimated at US$13.67 million. The cost-effectiveness of the GEF intervention, i.e., unit cost of CO2 mitigated under the Project, is US$0.876 per ton.

Table 1: Incremental Reasoning by Component

|  | Baseline project/ BAU scenario | GEF intervention/ GEF scenario | Global environmental benefits | Incremental cost |
| --- | --- | --- | --- | --- |
| National TAs | National policy directives to promote the development of multimodal freight transport and improve urban freight distribution - Local government accelerate the planning and investment in freight infrastructure and services - Some freight may be shifted to cleaner modes - Some urban freight trips may be consolidated and reduced | Policy/strategy/guideline to address impediments in policy, planning, operational, standardization, information for multimodal transport and urban freight distribution - Planning and investment informed by data and freight flow modal - China’s freight structure optimized - Railway/waterway capacity fully utilized - More long-haul freight shifted to railway and waterway - Urban freight efficiency improved - Urban freight trips consolidated, optimized and reduced | Indirect GHG emission reduction, contributing to China’s INDC goal | $3.15 million |
| Yantai pilot | Investment in logistics hubs and facilities, establishment of a trucking association for drop-and-pull, experienced third-party logistics companies to run the Bohai Bay multimodal freight business - Infrastructure built - Some freight traffic shifted from road to seaway - Third-party logistics operator engaged | Policies and incentives to attract more freight traffic to Yantai-Dalian seaway, credibility system of logistics operators, and business plan and operational plan to improve freight efficiency - Additional freight and trucks attracted to seaway - More operators join the trucking association - Successful operations of Bohai Bay multimodal freight business | Direct GHG emission reduction, from attracting additional freight trucks to use drop-and-pull compared to BAU (lifecycle CO2 emission reduction estimated to be 1.10 million tons) | $0.75 million |
| Weifang pilot | Construction of 4 urban freight consolidation centers - Freight consolidated in 4 centers being shipped to end customers | Data collection and analytics to provide urban distribution optimization solutions - Freight distribution in 4 centers optimized - Freight route optimized - Off-peak delivery plans - Introduction of clean energy vehicles | Direct GHG emission reduction, from reducing freight trip distance and increasing energy efficiency of freight vehicles compared to BAU | $0.55 million |
| Xiamen pilot | Construction of Xiamen port railway hub - Some freight shift from road to railway | Development of multimodal logistics information platform and optimizing port internal traffic - Multimodal freight transport efficiency improved - Easier access to shipping orders - More freight shifted from road to railway | Direct GHG emission reduction, from shifting more freight to railway compared to BAU | $0.70 million |
| Guangdong pilot | Construction of freight hubs and development of logistics information platform - Increased rural freight trips - Empty freight trucks from rural to urban | Pilot of urban-rural integrated distribution - Demand and supply matched between rural-to-urban freight and urban-to-rural empty return trucks - Rural freight consolidated - Return trucks replacing the individual villager’s rural freight trips | Direct GHG emission reduction, from avoiding rural-to-urban freight trips (lifecycle CO2 emission reduction estimated to be 0.23 million tons) | $0.75 million |
| Hubei pilot | Construction of the Yakou navigation-hydropower complex - Increased inland waterway capacity on Han River - Increased freight volume by waterway | Strategic plan for improving the integrated development of Han River inland waterway - Improved reliability of inland waterway transport on Han River - Improved efficiency of multimodal transport operation - More freight shifted from road to inland waterway | Direct GHG emission reduction, from shifting more freight to inland waterway compared to BAU | $0.30 million |
| Capacity Building |  | Workshops and trainings to promote the key policy/ strategy/guideline and disseminate the experiences gained from pilot projects - Quicker adoption of the policy/ guideline by stakeholders - Replication of the innovations from the pilots in other projects | Indirect GHG emission reduction, from replication effects | $2.05 million |

Table 2: Calculation for Yantai Pilot

| Annual number of trucks (*104) |  | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 | 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BAU | Roll on n roll off | 130.71 | 137.24 | 144.10 | 151.31 | 158.88 | 166.82 | 175.16 | 183.92 | 193.11 | 202.77 | 212.91 |
|  | DnP/Total ratio | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% | 2.10% |
|  | Drop n pull | 2.80 | 2.94 | 3.09 | 3.25 | 3.41 | 3.58 | 3.76 | 3.95 | 4.14 | 4.35 | 4.57 |
|  | Total trucks by seaway | 133.51 | 140.19 | 147.20 | 154.56 | 162.28 | 170.40 | 178.92 | 187.86 | 197.26 | 207.12 | 217.48 |
| GEF | Roll on n roll off | 130.71 | 137.24 | 144.10 | 151.31 | 158.88 | 166.82 | 175.16 | 183.92 | 193.11 | 202.77 | 212.91 |
|  | DnP/Total ratio | 5.28% | 7.00% | 10.00% | 13.00% | 13.00% | 13.00% | 13.00% | 13.00% | 13.00% | 13.00% | 13.00% |
|  | Drop n pull | 7.29 | 10.33 | 16.01 | 22.61 | 23.74 | 24.93 | 26.17 | 27.48 | 28.86 | 30.30 | 31.81 |
|  | Total trucks by seaway | 137.99 | 147.57 | 160.12 | 173.92 | 182.62 | 191.75 | 201.33 | 211.40 | 221.97 | 233.07 | 244.72 |
| Number of increased trucks by seaway |  | 4.48 | 7.39 | 12.92 | 19.36 | 20.33 | 21.35 | 22.42 | 23.54 | 24.71 | 25.95 | 27.25 |
| Emission factor for trucks (kg CO2/km) |  | 0.51 |  |  |  |  |  |  |  |  |  |  |
| Road transport distance (km) |  | 1400 |  |  |  |  |  |  |  |  |  |  |
| Direct emission reduction from road (ton) |  | 32004 | 52737 | 92253 | 138258 | 145171 | 152430 | 160051 | 168054 | 176456 | 185279 | 194543 |
| Number of ship trips per day (BAU) |  | 13 | 13 | 14 | 15 | 15 | 16 | 17 | 18 | 19 | 19 | 20 |
| Number of ship trips per day (GEF) |  | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Increased ship trips per day |  | 0 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 3 | 3 |
| Emission factor for ships (ton CO2/ton oil) |  | 3.15 |  |  |  |  |  |  |  |  |  |  |
| Fuel use per trip for ship (ton oil) |  | 18 |  |  |  |  |  |  |  |  |  |  |
| Increased CO2 by ship per year (ton) |  | 0 | 20695.5 | 20695.5 | 20695.5 | 41391 | 41391 | 41391 | 41391 | 41391 | 62086.5 | 62086.5 |
| Direct CO2 reduction per year (ton) |  | 32004 | 32041.5 | 71557.5 | 117562.5 | 103780 | 111039 | 118660 | 126663 | 135065 | 123192.5 | 132456.5 |
| Total direct CO2 reduction (ton) |  | 1104021.5 |  |  |  |  |  |  |  |  |  |  |

Table 3: Calculation for Guangdong Pilot

|  |  | 2020 | 2021 | 2022 | 2023 | 2024 | 2025 | 2026 | 2027 | 2028 | 2029 | 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BAU | Empty loaded rate (%) | 95% | 94% | 93% | 92% | 91% | 90% | 89% | 88% | 87% | 86% | 85% |
|  | Loaded return trips per month (4.2m) | 1100 | 1320 | 1540 | 1760 | 1980 | 2201 | 2421 | 2641 | 2861 | 3081 | 3301 |
|  | Loaded return trips per month (9.6m) | 400 | 480 | 560 | 640 | 720 | 800 | 879 | 959 | 1039 | 1119 | 1199 |
|  | Saved rural freight trips per month | 15198 | 18238 | 21277 | 24317 | 27356 | 30396 | 33436 | 36475 | 39515 | 42554 | 45594 |
| GEF | Empty loaded rate (%) | 95% | 90% | 80% | 65% | 65% | 65% | 65% | 65% | 65% | 65% | 65% |
|  | Loaded return trips per month (4.2m) | 1100 | 2201 | 4401 | 7702 | 7702 | 7702 | 7702 | 7702 | 7702 | 7702 | 7702 |
|  | Loaded return trips per month (9.6m) | 400 | 800 | 1599 | 2798 | 2798 | 2798 | 2798 | 2798 | 2798 | 2798 | 2798 |
|  | Saved rural freight trips per month | 15198 | 30396 | 60792 | 106386 | 106386 | 106386 | 106386 | 106386 | 106386 | 106386 | 106386 |
| Saved rural freight trips per month |  | 0 | 12158 | 39515 | 82069 | 79030 | 75990 | 72950 | 69911 | 66871 | 63832 | 60792 |
| Average distance (km) |  | 97 |  |  |  |  |  |  |  |  |  |  |
| Rural freight vehicle emission factor (kg CO2/km) |  | 0.32 |  |  |  |  |  |  |  |  |  |  |
| Direct CO2 reduction per month (ton) |  | 0 | 377 | 1227 | 2547 | 2453 | 2359 | 2264 | 2170 | 2076 | 1981 | 1887 |
| Direct CO2 reduction per year (ton) |  | 0 | 4529 | 14718 | 30569 | 29437 | 28305 | 27173 | 26040 | 24908 | 23776 | 22644 |
| Total CO2 reduction (ton) |  | 232099 |  |  |  |  |  |  |  |  |  |  |

Research on Fuel Consumption & Carbon Emission Factor of Road Freight Trucks. China Academic Journal. 1000-3703(2015)04-0037-04

The International Council on Clean Transport. Greenhouse Gas Emissions from Global Shipping, 2013-2015.