## Annex C	Estimate of Socio-economic and Environmental Benefits

1. Greenhouse gas emissions under SITRAMSS scheme

Reference is made to the description of SITRAMSS in Annex B. A preliminary assessment of the impact of SITRAMSS on GHG emissions and other atmospheric contaminants (OACs) was commissioned for VMT in 2015. It defines this first Corridor system as the “SITRAMSS Phase I influence area”.  In yearly figures, it involves a total of 161 millon displacements (passengers) on a total of 583 millon for the entire AMSS. The total number of buses and microbuses in the AMSS in 2012 was 2,257 and 2,033 respectively. Within the Phase I influence area this was 789 buses and 622 microbuses, totalling 1,411 units.

The assessment departs from the data used by the Consorcio Acciona Ingenieria for their “SITRAMSS Structuring and Basic Engineering Study (2012)”. The same reference year 2012 is used to determine emission factors and other parameters such as vehicle numbers, typical vehicle usage, fuel consumption, fuel properties, and more. This allows drawing a detailed baseline for 2012 including a projection of changes in the (conventional) bus routes. With the issuance of an updated Operational Plan for the public transport by VMT in 2014, this Baseline 2012 is adjusted to the new vehicle numbers and route projections (Baseline 2014). Of this system, only T I has been implemented so far, which since May 2017 is no longer working as a full BRT.

Baseline 2012 and 2014

The balance between the emissions of GHG and other atmospheric contaminants (OACs) should be compared between the SITRAMSS project and the reference situation in the area of incluence of SITRAMSS (system boundary). The assessment calculates the emissions of GHG and OACs in the AMSS and within the system boundaries for the years 2012 and 2014 according to the UNFCCC methodology (AM0031: Baseline Methodology for Bus Rapid Transit Projects, version 5.0). For OACs the same equations and procedures were applied as developed for the TransMilenio project in Bogota, Colombia in 2012.

In 2012 VMT envisaged to restructure 51 bus routes (1,411 units) and integrate these into the SITRAMSS. This figure was revised to 1,142 in the SITRAMSS Operational plan 2014 (649 buses and 493 microbuses), a reduction by 269 units. The difference is explained by the start of the construction of the SITRAMSS infrastructure (in 2013) and the adopted changes in bus routes. The Baseline 2012 involves the case that these 51 routes and 1,411 would have operated normally and SITRAMSS would not be implemented. Similarly, the 2014 Baseline considers the GHG emissions if the 45 routes and 1,142 units would operate normally. The following table represents the two baseline situations.

| SITRAMSS – Transport Figures under Baseline Situation in 2012 and 2014 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Unit | Total AMSS |  | SITRAMSS Phase I area |  |
|  |  | 2012 | 2014 | 2012 | 2014 |
| Number of buses | - | 2,257 | 2,296 | 789 | 649 |
| Number of microbuses | - | 2,033 | 1,862 | 622 | 493 |
| Total number of units | - | 4,290 | 4,158 | 1,411 | 1,142 |
| Total annual distance by buses | km/yr | 87,318,816 | 88,827,648 | 33,589,624 | 26,566,815 |
| Total annual distance by microbuses | km/yr | 111,636,096 | 102,246,144 | 4,410,379 | 33,851,352 |
| Number of passengers transported by buses | pp/yr | 374,831,275 | 381,308,200 | 131,033,175 | 107,782,675 |
| Number of passengers transported by microbuses | pp/yr | 220,387,365 | 201,850,110 | 67,427,910 | 53,443,665 |
| Total number of passengers transported | pp/yr | 595,218,640 | 583,158,310 | 198,461,085 | 161,226,340 |
| Diesel fuel consumption | mln l/yr | 48.5 | n/d | 19.0 | 16.9 |

Based on typical fuel consumption rates for buses and microbuses and their recorded mileage in the AMSS, total diesel fuel consumption by public transport in the AMSS (2012) is estimated at 48.5 mln l (22 mln l buses and 26.5 mln microbuses). This figure is somewhat higher than the registered amount of fuel commercialized in the AMSS (46.9 mln l). Similarly, the diesel consumption of the vehicles in the influence area of SITRAMSS Phase 1 are calculated at 19.0 mln l (8.5, resp 10.5 mln l for buses and and microbuses). Buses account for 45% of total and microbuses 55% and is fairly the same in the influence area, and the overall AMSS.

The emissions of GHGs and OACs under the Baseline 2012 and 2014 are presented in the next table.

| SITRAMSS – Emissions under Baseline Situation in 2012 and 2014 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Unit | Total AMSS |  | SITRAMSS Phase I area |  |
|  |  | 2012 | 2014 | 2012 | 2014 |
| Greenhouse Gas (GHG) Emissions |  |  |  |  |  |
| CO2, N2O | tCO2e/yr | 149,904 | 144,899 | 58,650 | 45,532 |
| Other atmospheric contaminants (OAC) Emissions |  |  |  |  |  |
| CO | ton/yr | 3,002 | 2,890 | 1,176 | 912 |
| HC |  | 299 | 288 | 117 | 91 |
| Nox |  | 978 | 941 | 383 | 297 |
| SO2 |  | 280 | 270 | 110 | 85 |
| PM10 |  | 247 | 238 | 85 | 75 |

The study observes that the Baseline 2014 is probably the one that best represents the situation without SITRAMSS operating. However, its reference value is somewhat compromised because works on SITRAMSS were already in progress impacting on local bus routes and passenger rates. Methodologically, the input parameters are 2012 values which by itself are sometimes best estimates, hence the calculations may made more accurate using better input data once these are available. The input parameters and emission factors used are summarized at the end of this annex.

SITRAMSS alternative scenario 2012 and 2014

The implementation of SITRAMSS Corridor I involves the following scenarios:

(1) A “non-optimized 2012 system” based on a reduction of vehicle number to 967 as follows: 368 operational buses composed of a mix of 85 new buses and 283 re-used units, and 502 microbuses (82 new microbuses and 420 re-used ones); plus 35 articulated and 62 short SITRAMSS buses (padrones). This alternative allows for maintaining existing routes to a large extent continues the use of conventional buses by adding new units to this fleet. Total diesel consumption would be 16,745,389 l/yr.

(2) An “optimized 2012 system” which assumes a total of 683 vehicles operating: 198 re-used buses and 183 re-used microbuses, plus 35 articulated and 267 short SITRAMSS buses (padrones).  Compared to the first scenario, this alternative encompasses the replacement of large number of conventional buses by SITRAMSS “padrones” offering high-quality service. Total diesel consumption would be 13,608,707 l/yr.

(3) A “2014 system” based on a reduction of the 1,142 units to 672 as follows: 340 re-used buses and 332 re-used microbuses, plus 27 articulated and 138 short SITRAMSS buses (padrones). The 2014 scenario assumes that feeder (T) and pre-feeder (P) routes will replace the current bus system as follows: T I (Soyapango); T2 (Ilopango); P1 (Soyapango to Salvador del Mundo) and P2 (Ilopango – Salvador del Mundo via Agua Caliente). In 2015, route T I has been implemented. The advantage of this scenario is that it avoids investment in new conventional buses while it encompasses less SITRAMSS units than under alternative 2. Total diesel consumption would be 10,470,694 l/yr.

The next table provides a breakdown of GHG emissions as made by the assessment study for the Baseline 2012 and 2014 baseline and the three depicted scenarios. It further shows the the estimated GHG emission reductions compared to the Baseline 2012 and 2014. For the 2014 case, it  specifies the contributions of the different route  systems to the total  GHG reduction. The estimates are based on the assumption that the number of users of SITRAMSS grows by 3.5% in year 1, 5.5% year 2, 8.0% year 3. These data are based on observations of the BRT system in Guatemala City. For subsequent years, a figure of 0.24% is taken corresponding to the observed demand growth in the current public transport system in the AMSS.

| SITRAMSS – Greenhouse Gas Emissions under Baseline and Alternative Scenarios for 2012 and 2014 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | YEAR |  |  |  |  |  |  |  |  |  |  |
|  | 2015 | 2016 | 2017 | 2018 | 2019 | 2020 | 2021 | 2022 | 2023 | 2024 | TOTAL |
| Annual GHG Emissions Scenarios 1, 2 and 3 (tCO2e) |  |  |  |  |  |  |  |  |  |  |  |
| Base Line 2012 | 57,322 | 56,885 | 56,451 | 56,021 | 55,594 | 55,170 | 54,750 | 54,332 | 53,918 | 53,507 | 553,950 |
| Base Line 2014 | 45,185 | 44,840 | 44,498 | 44,159 | 43,823 | 43,488 | 43,157 | 42,828 | 42,501 | 42,177 | 436,656 |
| SITRAMSS 1 (2012) | 44,495 | 44,050 | 43,610 | 43,174 | 42,742 | 42,314 | 41,891 | 41,472 | 41,058 | 40,647 | 425,453 |
| SITRAMSS 2 (2012) | 36,526 | 36,161 | 35,799 | 35,441 | 35,087 | 34,736 | 34,388 | 34,044 | 33,704 | 33,367 | 349,253 |
| SITRAMSS 3 (2014) | 27,822 | 27,544 | 27,269 | 26,996 | 26,726 | 26,459 | 26,194 | 25,932 | 25,673 | 25,416 | 266,031 |
| Annual GHG Emission reductions Scenarios 1 and compared to Base Line 2012 (tCO2e) |  |  |  |  |  |  |  |  |  |  |  |
| SITRAMSS 1 (2012) | 12,827 | 12,835 | 12,841 | 12,847 | 12,852 | 12,856 | 12,859 | 12,860 | 12,860 | 12,860 | 128,497 |
| SITRAMSS 2 (2012) | 20,796 | 20,724 | 20,652 | 20,580 | 20,507 | 20,434 | 20,362 | 20,288 | 20,214 | 20,140 | 204,697 |
| Annual GHG Emission reductions Scenario 3 compared to Base Line 2014 (tCO2e) |  |  |  |  |  |  |  |  |  |  |  |
| SITRAMSS 3 (2014) | 17,363 | 17,296 | 17,229 | 17,163 | 17,097 | 17,029 | 16,963 | 16,896 | 16,828 | 16,761 | 170,625 |
| Contribution of Annual GHG Emission Reductions per route compared to Base Line 2014 (tCO2e) |  |  |  |  |  |  |  |  |  |  |  |
| SITRAMSS route T1 | 2,224 | 2,302 | 2,428 | 2,622 | 2,629 | 2,635 | 2,641 | 2,648 | 2,654 | 2,660 | 25,443 |
| SITRAMSS feeders T2, P1, P2 | 8,086 | 8,370 | 8,829 | 9,536 | 9,559 | 9,582 | 9,605 | 9,627 | 9,651 | 9,674 | 92,519 |
| Conventional bus reductions | 7,053 | 6,624 | 5,972 | 5,005 | 4,909 | 4,812 | 4,717 | 4,621 | 4,523 | 4,427 | 52,663 |

Based on the 2012 data, it is possible to estimate the annual fuel consumption under the Baseline 2014 and the alternative scenario SITRAMSS 3. Specific fuel consumption values are defined as in the table below.

| SITRAMSS – Fuel consumption under 2014 and Alternative Scenarios |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle type | Number | total distance | total per unit | Specific fuel use | Annual fuel use |
|  | (-) | (km/yr) | (km/yr) | (l/km) | (l/yr) |
| Annual Diesel Fuel Consumption2014 Baseline |  |  |  |  |  |
| Buses | 649 | 26,566,815 | 40,935 | 0.32 | 8,501,381 |
| Microbuses | 493 | 33,851,312 | 68,664 | 0.25 | 8,462,828 |
| TOTAL | 1,142 | 60,418,127 |  |  | 16,964,209 |
| Annual Diesel Fuel consumption under SITRAMSS 3 Scenario (2014) |  |  |  |  |  |
| SITRAMSS articulated (T1) | 27 | 1,657,049 | 61,372 | 0.5 | 828,525 |
| SITRAMSS short (T2, P1, P2) | 138 | 8,142,468 | 59,003 | 0.37 | 3,012,713 |
| Re-used buses | 340 | 11,834,172 | 34,806 | 0.32 | 3,786,935 |
| Re-used microbuses | 332 | 11,370,087 | 34,247 | 0.25 | 2,842,522 |
| TOTAL | 837 | 33,003,776 |  |  | 10,470,694 |
| Idem, per SITRAMSS Feeder Route (2014) |  |  |  |  |  |
| T2 | 36 | 2,701,514 | 75,042 | 0.37 | 999,560 |
| P1 | 61 | 3,346,616 | 54,863 | 0.37 | 1,238,248 |
| P2 | 41 | 2,094,338 | 51,081 | 0.37 | 774,905 |
| TOTAL | 138 | 8,142,468 | 59,003 |  | 3,012,713 |
| Annual Fuel Savings under SITRAMSS 3 Scenario (2014) |  |  |  |  |  |
| Fuel savings (l/yr) |  |  |  |  | 6,493,514 |
| Monetary savings (US$/yr) (@ 0.93 USD/l) |  |  |  |  | 6,046,871 |
| Annual GHG Emission Reductions under SITRAMSS 3 Scenario (2014) |  |  |  |  |  |
| GHG emissions reductions (tCO2e/yr) (EF: 2.661 kg CO2e/l) |  |  |  |  | 17,279.24 |

As seen in the table, total fuel uses are reduced by 6.5 mln litre per year with associated savings of the order of US$ 6 mln per year. Fuel savings and diesel emission factor provide a simple estimate for annual GHG emission reductions for the base year, to be compared with the 17,363 tCO2e/yr in the previous table.

| SITRAMSS 3 – Fuel use per route type |  |  |
| --- | --- | --- |
| Vehicle type | Annual fuel use |  |
|  | (l/yr) | (%) |
| SITRAMSS articulated (T1) | 828,525 | 8% |
| SITRAMSS short (T2, P1, P2) | 3,012,713 | 29% |
| Re-used buses | 3,786,935 | 36% |
| Re-used microbuses | 2,842,522 | 17% |
| TOTAL | 10,470,694 | 100% |

Baseline scenario 2018

The SITRAMSS 3 (2014) scenario implies a transformation of the conventional bus network in the influence area of SITRAMSS Corridor I into a feeder system. This has not been achieved and, as of 2018, T I operates with only 16 articulated buses end 21 padrones (37 units). The Baseline 2018 therefore consists of this T I plus a number of buses and microbuses that has evolved from the Baseline 2014. Since demand for public transport has not decreased over since 2018, the conventional buses in circulation will probably be similar to the Baseline 2014 situation. Factual information about the current situation is not available.

Since demand of public transport in the area has not decreases and the conventional bus system remains based competitive, it seems reasonable to assume that most or all of the buses and microbuses from the Baseline 2014 are still in service.  Reportedly, conventional buses now run in parallel to the articulated SITRAMSS buses along the T I stretch since May 2017 with the SITRAMSS buses operating at similar speeds. The bus operator has limited the deployment of the short buses to avoid incurring in high operational losses. Based on this information, the current situation (Baseline 2018) can be approximated as in the next table.

| SITRAMSS – Estimated Baseline 2018 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle type | Number | total distance | total per unit | Specific fuel use | Annual fuel use |
|  | (-) | (km/yr) | (km/yr) | (l/km) | (l/yr) |
| Annual Diesel Fuel Consumption2014 Baseline |  |  |  |  |  |
| SITRAMSS articulated (T1) | 16 | 981,955 | 61,372 | 0.5 | 490,977 |
| Buses | 649 | 26,566,815 | 40,935 | 0.32 | 8,501,381 |
| Microbuses | 493 | 33,851,312 | 68,664 | 0.25 | 8,462,828 |
| TOTAL | 1,158 | 61,400,082 |  |  | 17,455,186 |
| Annual GHG Emissions under Baseline 2018 |  |  |  |  |  |
| GHG emissions (tCO2e/yr) (EF: 2.661 kg CO2e/l) |  |  |  |  | 46,448 |

As such, fuel consumption and GHG emissions have increased slightly compared to the Baseline 2014 and at a fixed number of passengers, passenger density per unit will have dropped slightly.

SITRAMSS alternative scenario 2018

The alternative scenario for the GEF project is based on the assumption that combined action will enable the authorities to push forward a reorganization of bus routes towards the SITRAMSS 3 scenario. The absence of investment finance committed to the Project inhibits the generation of direct GHG benefits by procurement of new SITRAMSS vehicles. At PIF stage, such investment was assumed. However, “soft action” is expected to translate into a reduction of conventional buses and microbuses as follows. The already purchased 21 “padrons” are 15% of the anticipated number of 138 units. Without fresh investment, re-routing will arguably result in a proportional reduction in conventional units and their annual usage rates.

- N_buses = N_baseline-15%*(N_baseline – N_SITRAMSS 3) = 649 – 15%*(649-340) = 602; a reduction of 47 units; D_buses = 40,935 – 15%*(40,935-34,906) = 40,031 km/yr;

- N_micro = 493 – 15%*(493-332) = 468; a reduction of 25 units. D_micro = 68,664 – 15%*(68,664-34,247) = 63,501 km/yr.

Then the SITRAMSS 2018 scenario will look as follows:

| SITRAMSS – Estimated scenario 2018 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle type | Number | total distance | total per unit | Specific fuel use | Annual fuel use |
|  | (-) | (km/yr) | (km/yr) | (l/km) | (l/yr) |
| Annual Diesel Fuel consumption under SITRAMSS 3 Scenario (2014) |  |  |  |  |  |
| SITRAMSS articulated (T1) | 16 | 981,955 | 61,372 | 0.5 | 490,977 |
| SITRAMSS short | 21 | 1,239,071 | 59,003 | 0.37 | 458,456 |
| Re-used buses | 602 | 24,098,451 | 40,031 | 0.32 | 7,711,504 |
| Re-used microbuses | 468 | 29,718,679 | 63,501 | 0.25 | 7,429,670 |
| TOTAL | 1,107 | 56,038,156 |  |  | 16,090,608 |
| Annual Fuel Savings under SITRAMSS 3 Scenario (2014) |  |  |  |  |  |
| Fuel savings (l/yr) |  |  |  |  | 1,364,578 |
| Monetary savings (US$/yr) (@ 0.93 USD/l) |  |  |  |  | 1,270,718 |
| Annual GHG Emission Reductions under SITRAMSS 3 Scenario (2014) |  |  |  |  |  |
| GHG emissions reductions (tCO2e/yr) (EF: 2.661 kg CO2e/l) |  |  |  |  | 3,631 |

Over a 10-year period, simple extrapolation yields direct GHG emission reductions of the order of 36 kton CO2e.

The GEF Project strives at establishing the enabling conditions: an adequate regulatory framework, a transparent business model, enforcement of the reorganization of the conventional buses and broad social acceptance. As such, upscaling to implement SITRAMSS Corridor I is claimed as post-project investment. The attained GHG emission reductions are then as estimated for scenario SITRAMSS 3 (17 kton CO2e per year, or 170 kton CO2e over a 10-year period). Summarizing:

- Direct GHG emission reductions are 36 kton CO2e (over 10-year lifetime);

- Indirect, post-project GHG emission reductions are 170 – 36 = 134 kton CO2e.

2. Greenhouse gas reductions from energy conservation in the public sector

The present assessment is based on a study commissioned by CNE and UNDP in 2014 under the GEF-4 EEPB project. This study explores the potential for a NAMA for public buildings, public lighting and vehicles in use in the public sector, and provides a description of the baseline situation and the effect of a series of specified interventions. Based on the latter, it is concluded that public vehicle use is of secondary relevance for municipalities as most are operated by central Government entities, most notably the National Civil Police (PNC).  The buildings are classified per Government entity but a precise classification of municipal buildings could not be made either at that time and is neither available today. For public lighting a fairly detailed baseline was produced as the inventory could be correlated with billing data from the electricity distribution companies.

It is noted that in 2014 no LED lighting was implemented; visual observation as of June 2018 however makes evident the use of LED lighting along major highways (San Salvador – Comalapa) and avenues in the AMSS.  However, one can assume that little has been changed in the baseline for municipalities since 2013 as they lack access to finance for upgrading of public lighting.

Public buildings

According to the 2011 Census, the public administration counted 6,542 buildings covered by a total of 7,559 electricity connections. The buildings are distributed among 15 government sectors and 136 institutions. The total consumption was estimated at 315,000 MWh/yr. Only five sectors make up 76% of total energy consumption: Education, Public Health, Work and Social Prevention, National Defence, and Autonomous Entities. The major institutions within these sectors are the National Water and Sewage Administration (ANDA), the hospitals, the Ministry of Education (which operates school buildings) and the Autonomous Port Executive Commision (CEPA). In fact, ANDA accounts for 34% of total energy consumption (mainly for water pumping) and is the only one sourcing energy from the wholesale market.

Recent figures provided by CNE indicate that, for the Central Government only, total energy consumption has remained fairly stable: 243,800 MWh/yr (2013) compared to 249,870 MWh/yr (2016). This suggests that also the overall baseline figure would not have changed significantly since 2014. Partly due to the reduction in oil prices in that period, total billing dropped from US$ 53.7 mln (2013) to US$ 35.5 mln (2016) however. In 2016, the unit cost for electricity purchased by the Central Government was:

= 35,500,000 US$/249,870 MWh = 142 USD/MWh.

Electricity billing in larger buildings (above 300 kVA) is by measurement in the medium-voltage circuit (before the transformer) and in smaller buildiings (below 300 kVA) is by one or more electricity meters in the low-voltage circuit. The most important parameters composing the electricity bill are the cost charge per energy consumed and the distribution charge, which depend on the contracted power.

The following table presents an overview of electricity and fuel consumption in public buildings per government sector, as adapted from the previously mentioned NAMA study.

| Fuel and electricity consumption per type of public end user |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type of Energy |  |  |  | Total |  |
|  | LPG | Diesel oil | Gasoline | Electric energy |  |  |
| Government Sector | (Mwh/yr) |  |  |  | (Mwh/yr) | (%) |
| Education | 9,605 | 1,523 | 337 | 42,187 | 55,469 | 16% |
| National Defense | 22,896 | 1,907 | 1,151 | 23,643 | 50,270 | 14% |
| Public Health | 14,151 | 12,593 | 698 | 44,078 | 72,183 | 21% |
| Work and Social Prevention | 6,709 | 16,826 | 291 | 37,195 | 61,021 | 17% |
| Autonomous entities | 0 | 767 | 151 | 41,632 | 42,551 | 12% |
| Others | 129 | 5,221 | 4,663 | 59,319 | 69,332 | 20% |
| TOTAL | 53,490 | 38,837 | 7,291 | 248,054 | 350,826 | 100% |
|  | 15% | 11% | 2% | 71% | 100% |  |

Similarly, then next table provides a breakdown of electricity and fuel consumption per final energy use.

| Fuel and electricity consumption per final energy use in public buildings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type of Energy |  |  |  | Total |  |
|  | LPG | Diesel oil | Gasoline | Electric energy |  |  |
| Final Energy Use | (Mwh/yr) | (Mwh/yr) | (Mwh/yr) | (Mwh/yr) | (Mwh/yr) |  |
| Cooking | 51,140 | 0 | 0 | 0 | 53,630 | 15% |
| Lighting | 0 | 0 | 0 | 47,245 | 47,245 | 13% |
| Climatization | 0 | 0 | 0 | 112,442 | 112,442 | 32% |
| Office equipment | 0 | 0 | 0 | 31,480 | 31,480 | 9% |
| Transport | 1,179 | 13,361 | 6,779 | 0 | 21,319 | 6% |
| Other uses | 180 | 547 | 512 | 56,887 | 58,126 | 17% |
| Electricity generation | 0 | 570 | 0 | 0 | 570 | 0% |
| Steam production | 991 | 24,361 | 0 | 0 | 26,014 | 7% |
| TOTAL | 53,490 | 38,839 | 7,291 | 248,054 | 350,826 | 100% |
|  | 15% | 11% | 2% | 71% | 100% |  |

The NAMA study proposes a number of interventions and provides a cost estimate. For the purpose of the present GHG estimate, it is assumed that the same “mix” is applied in the first batch of EE projects to be implemented under the GEF project (using US$ 3 mln cofinancing resources for investment in public buildings).

| NAMA Public Buildings |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Intervention Key Data |  |  |  |
|  | Investment |  | Electric energy savings | GHG emission reductions |
| Proposed Measures | project | US$ (mln) | Mwh/yr | tCO2eq/yr |
| - Replacement of 20% of interior and exterior lighting and retrofitting of 50%; - Change of refrigerants in 30% of climatization equipment; - Substitution of 30% of standard electric motors by EE ones; - Installation of thermal insulation of windows in 33% of buildings; and - Installation of PV panels (1kWp) in 20% of buildings. | NAMA | US$ 31.1 | 18,247 | 15,129 |
|  | GEF | US$ 3.0 | 1,760 | 1,459 |

It is envisaged to implement the measures under an energy management plan. Experience woldwide shows that the existence of such plan and the adoption of improved user habits, rationalization of building use and occupance, and preventive maintenance, delivers tangible energy savings. Here a guiding figure of 15% is used.

| GEF Project - Electricity savings and GHG reductions from investments and energy management |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | Investment GEF project | Energy management | Total |
| Electric energy savings | (Mwh/yr) | 1,760 | 264 | 2,024 |
| GHG emission reductions | (tCO2eq/yr) | 1,459 | 219 | 1,678 |

Over a 10-year period, total GHG reductions amount to 16,780 tCO2 and energy savings 20,240 MWh. At a price of 140 USD/MWh, the associated annual monetary savings would be USD 283,000, implying a simple payback time of 10.6 year.

Public lighting

The total number of public light units (luminaires) in the country is approximately 187,000, distributed over the 262 municipalities. The public lighting service in El Salvador is a responsibility of the municipalities. The municipalities are in charge of planning, implementation and maintenance of this service and are responsible for paying the distribution companies for the electricity consumed. The municipalities can implement the public lighting service directly; however, it is usually outsourced to the electricity distribution companies, which are: CAESS, DELSUR, CLESA, EEO, DEUSEM, B&D, EDESAL y ABRUZZO.

The necessary funds are generated through the Municipal Public Lighting Tax (Tasa Municipal de Alumbrado Público - TMAP), which is set annually by each Municipal Council. The TMAP can be collected directly by each municipality or by the distribution company under an agreement, in which case the TMAP is included in the electricity bill of the consumers. The tariff has not hold pace with the actual costs of the public lighting service which has led to a deterioration of the service. The lack of funding represents a barrier for implementing energy efficiency measures such as a change in technology. It is important to note that the tariffs do not cover maintenance, installation and substitution of luminaires.

An assessment of the total stock of public light units in the country learns, that 31% is located in only 5 municipalities and, at the other side of the spectrum, there are 200 municipalities which on average count only 234 luminaires (see next table).

| Distribution of public light units per municipality |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number of light units | Number of municipalities | Name of municipality | Number of light units |  | Average number of light units per municipality |
| (range) | (-) | (-) | (-) | (%) | (-) |
| >10,000 | 1 | San Salvador | 25,309 | 13.5% | 25,309 |
| 5,500-10,000 | 4 | Santa Ana, San Miguel, Soyapango, Santa Tecla | 32,803 | 17.5% | 8,201 |
| 1,000-5,500 | 57 | 57 municipalities | 82,280 | 44.0% | 1,444 |
| <1,000 | 200 | remainder | 46,797 | 25.0% | 234 |
| TOTAL |  |  | 187,189 | 100.0% |  |

The NAMA study made an inventory of the lighting technologies in use, which range for vapor discharge lamps (mercury and high-pressure sodium) and, in the smaller municipalities, mostly fluorescent lights. The NAMA study proposes the replacement of existing public light by the most efficient technology (LED) and provides a cost estimate. For the purpose of the present GHG estimate, it is assumed that the same “mix” of measures is applied in the first batch of EE projects to be implemented under the GEF project (using US$ 2 mln cofinancing resources for investment in public lighting). Then, the energy savings and GHG emission reductions can be estimated directly.

| NAMA Public Lighting |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Intervention Key Data |  |  |  |  |
|  | Investment |  | Number of public light units | Electric energy savings | GHG emission reductions |
| Proposed Measure | project | US$ (mln) | - | Mwh/yr | tCO2eq/yr |
| Replacement of existing public light by the most efficient technology (LED of various capacities) | NAMA | US$ 100 | 184,533 | 87,838 | 68,635 |
|  | GEF | US$ 2.0 | 3,680 | 1,752 | 1,369 |

Over a 10-year period, total GHG reductions amount to 13,690 tCO2 and energy savings 17,520 MWh. It is noted that that the tariff for public lighting (approx 200 USD/MWh, depending on the local electricity distribution company) is higher than for public buildings. At this tariff, the associated annual monetary savings would be USD 350,000, implying a simple payback time of 5.7 year.

3. Summary of expected economic and environmental (GHG) benefits

| Summary of expected economic and environmental (GHG) benefits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Direct benefits |  |  |  |  | Indirect (replication) |
| Technology | energy savings |  | monetary savings |  |  |  |
|  | (MJ/yr) | (MWh/yr) | (US$/yr) | (tCO2e/yr) | (tCO2e) | (tCO2e) |
| 1. Mobility | 43,666,496 | 12,130 | 1,270,718 | 3,631 | 36,310 | 134,000 |
| 2. Public building |  | 2,024 | 283,000 | 1,678 | 16,780 | 33,560 |
| 3. Public lighting |  | 1,752 | 350,000 | 1,369 | 13,690 | 27,370 |
| Total |  | 15,906 | 1,904,405 | 6,678 | 66,780 | 194,930 |

For EE in public buildings and public lighting, a replication factor of 2.0 is assumed for investment post-project through the FIDEnergetica trust fund.

Informe Final “Estimación Preliminar acerca del Potencial de Reducción de Emisiones de Gases de Efecto Invernadero y otros Contaminante Atmosféricos” by BioSistemas SA de CV, Juan Pablo Domínguez Miranda e.a, 5 January 2015.

Estudio para la Estructuración e Ingeniería Básica del SITRAMSS (Consorcio Acciona Ingeniería, et al., 2012 and its annexes.

Determined according to table “Fuel Consumption 2014”.

Informe BioSistemas SA de CV, p.44-45.

NAMA en el uso final de la energía del sector gubernamental – Estudio sobre los sub-sectores de alumbrado público, edificios públicos y transporte nacional, project UNDP/CNE/00075672, Alberto Galante Marcos, Luca Lo Re, e.a. Perspectives GmbH, Hamburg Germany, April 2014.

Adapted from: Perspectives GmbH, Table 11, p.31.

Adapted from: Perspectives GmbH, Table 10, p.30.

Adapted from: Perspectives GmbH, Table 35, p.57 (scenario 2).

Adapted from: Perspectives GmbH, Table 4, p.22.

Adapted from: Perspectives GmbH, Table 31, p.51 (scenario 1).